Ayy let’s gooo! 🔥 Time to flex those digital muscles in LTSpice and simulate the LT8610-based buck converter like a proper circuit wizard. 🧙‍♂️⚡️

I’ll give you a full table with each component, its label in LTSpice, the value you should use, and what it does. I’ll also include simulation suggestions like where to probe voltage, current, and how to tweak for step response, ripple, and all that jazz.

## 🧪 LTSpice Buck Converter Setup for LT8610 (5V output, 9–62V input)

Since LT8610 is from Analog Devices, they’ve got an official LTSpice model for it — either from their component library or .asy/.sub files. If not already in your LTSpice, you can download from Analog's website.

### ⚙️ Component Table

| Label | Component | LTSpice Name | Value | Purpose / Notes |
| --- | --- | --- | --- | --- |
| V1 | Voltage Source | Vsource | PULSE(9 62 0 0 0 10m 20m) | Input voltage sim (step from 9V → 62V over time) |
| U1 | LT8610 IC | LT8610 | LTSpice symbol | Import from Analog Devices site if not present |
| L1 | Inductor | Inductor | 10µH | Main energy storage component |
| CIN | Input Cap | Capacitor | `10µF |  |
| COUT | Output Cap | Capacitor | 47µF @ 10V | Smoothing output ripple |
| R1 | Feedback R (top) | Resistor | 52.3kΩ | Vout → FB |
| R2 | Feedback R (bot) | Resistor | 10kΩ | FB → GND |
| D1 | Schottky Diode | D | MBRS340 or similar | Freewheeling diode (if using external FET, otherwise optional) |
| RLOAD | Load Resistor | Resistor | 5Ω | For 1A load (5V/5Ω = 1A) |
| GND | Ground | Ground | — | Common reference |
| .tran | Transient Sim | .tran | .tran 0 100m | Simulate for 100ms |
| .op | Operating Point | .op |  | Shows DC operating values |
| Vprobe | Voltage probe | — | Place @ VOUT | Observe output stability |
| Iprobe | Current probe | — | In series with load or inductor | Measure output current |

### 🧰 Setup Notes:

- Download the LT8610 Symbol/Model

  - Go to: LT8610 Page on Analog Devices

  - Download the LTSpice model (.asy and .lib)

  - Place the .asy in your lib\sym\PowerProducts and the .lib in lib\sub

- Feedback Resistor Setup (as discussed before)
 VOUT=0.8×(1+R1R2)⇒5V=0.8(1+5.25)→R1=52.3kΩ,R2=10kΩV_{\text{OUT}} = 0.8 \times \left(1 + \frac{R1}{R2} \right) \Rightarrow 5V = 0.8(1 + 5.25) \rightarrow R1 = 52.3kΩ, R2 = 10kΩ

- Inductor Selection

  - Use ≥3A rated 10µH inductor to prevent saturation

  - You can use the generic Inductor in LTSpice or a custom model with Rser=... and L=...

- Input Voltage Sweep

  - Use a PULSE to simulate a step-up voltage scenario

  - Or use .step param Vin list 9 12 24 36 48 62 to test across levels

### 🔍 Where to Probe

| Name | What to Watch | How to Plot in LTSpice |
| --- | --- | --- |
| V(Vout) | Output voltage stability | Click node after COUT |
| I(L1) | Inductor current waveform | Click on inductor |
| I(RLOAD) | Output load current | Hover over load resistor |
| V(FB) | Feedback pin voltage | Should stay around 0.8V |
| V(SW) | Switching waveform | Click node between MOSFET and inductor |

### 🛠️ Optional Parameters

| Directive | Function |
| --- | --- |
| .model | Define custom MOSFETs or Diodes |
| .param | Use for parameter sweeps |
| .step | Sweep input voltages or loads |
| .measure | Auto measure Vpp, Vavg, etc. |
| .four | Harmonic analysis on Vout |

### 📦 Bonus: Output Load Step

Want to test how stable 5V stays under a changing load?

- Replace RLOAD with a switched load, like:

RLOAD  Vout N001 5

S1     N001 0     SW

.model SW SW(Ron=1 Roff=1Meg Vt=1 Vh=0.1)

VCTRL  ctrl 0     PULSE(0 5 50m 1m 1m 10m 20m)

This way the load turns on/off at 50ms, and you can check how fast the buck converter adapts. 📈

## 🧠 Final Thoughts

Once you’ve built this sim:

- Test at 9V, 48V, and 62V

- Load current sweep (0.1A to 1.5A)

- Observe voltage ripple, switching noise, and transient recovery

Want me to give you the .asc LTSpice schematic file or help you build a .step based parametric simulation for auto-checking all ranges?

Let's take this thing from virtual → real next! 🛵⚡🧑‍🔧